'The main TestEvent Macro to call.  It's used to call one of three

'separate Test Macros based on the measurement mode of the PMM.

'*********************************************************************

Function FlukeCalSledCalibrationAdjust(iT, iL, iC, iP, cTest, cConfig)

' Determine which macro to run.  There are five Meas Mode codes:

'0 = Absolute

'1 = Gauge

'2 = Abs by Atms

'3 = Differential (Bi-Directional)

'4 = ???

'5 = Negative Gauge

Select Case cTest.TestPrsMeasMode

Case 0:

Call FlukeCalSledAbsolute(iT, iL, iC, iP, cTest, cConfig)

Case 1:

Call FlukeCalSledGauge(iT, iL, iC, iP, cTest, cConfig)

Case 3:

Call FlukeCalSledDifferential(iT, iL, iC, iP, cTest, cConfig)

End Select

End Function

Function FlukeCalSled_Dwell(dwell)

cDebug.LogStatus "Dwell for " & dwell & "s"

tStart = timer

Do

cCOMPASS.TimeDelay 1

td = CInt(dwell-time_Difference(tStart))

cCOMPASS.StatusDisplay "Dwell..." & td

If cCOMPASS.SystemAbort Then Exit Function

If td <= 0 Then Exit Do

Loop Until False

End Function

Function FlukeCalSled_WaitForReady(isZero, timeout)

cDebug.LogStatus "Wait for Ready: timeout=" & timeout

tStart = Timer

'The Do Loop tells COMPASS to look at the logic output of the UseReady method.  The logic True / False

'can be queried by using the .Ready parameter.  The return from cConfig.SetPres(1).Ready is either TRUE or FALSE.

Do

cCOMPASS.TimeDelay 2   'delay controller

cCOMPASS.StatusDisplay process & ": Waiting for Ready.........."

If cCOMPASS.SystemAbort Then Exit Function

If Time_Difference(tStart) > timeout Then

cDebug.LogStatus "*******TIMEOUT WAITING for Ready****"

cCOMPASS.StatusDisplay process & ": ............TIMEOUT Waiting for Ready.........."

cCOMPASS.TimeDelay 5 'delay to show above message

If (cCOMPASS.cConfig.DUTPrs(1).RangeMain.MeasMode = 0) And (isZero = 1) Then

cDebug.LogStatus "Timeout for absolute 0, as good as it will get"

Exit Do 'absolute 0, as good as possible

End If

Exit Function

End If

'If the SetPrs device supports the UseReady concept then loop until the .Ready parameter becomes TRUE or,

'If the SetPrs device doesn't support UseReady then look at if the RefPrs device does.  The presumption

'is that either the controller or the reference device will support the UseReady method.  Exit the loop

'once one of these two devices indicates Ready.

If cConfig.SetPrs(1).RangeMain.Useready Then

cDebug.LogStatus "Wait for ready...SetPrs.Ready=" & cConfig.SetPrs(1).Ready

If cConfig.SetPrs(1).Ready Then Exit Do

Else

cDebug.LogStatus "Wait for ready...RefPrs.Ready=" & cConfig.RefPrs(1).Ready

If cConfig.RefPrs(1).Ready Then Exit Do

End If

Loop

FlukeCalSled_WaitForReady = True

End Function

Function FlukeCalSled_Date()

yy = right(Year(Date),2)

dd = Right(String(2, "0") & Day(Date), 2)

mm = Right(String(2, "0") & Month(Date), 2)

FlukeCalSled_Date = mm & "/" & dd & "/" & yy

End Function