Additel 671

27 Sept 2016

- Requires a non-traditional termination character – a decimal “0”

- Must use a GetCommand macro to append “0” to end of the command string(s).

- Has a High and Low range.

The Command and Response Terminator fields should be empty (delete):

Two Outputs – both are of traditional configuration; Raw-to-Final output is <Same>.  Use Tolerance of ±0.02% Span.   The specifics of each output are in the Edit Commands subsection.

Edit Commands, “Hi P Sensor”:

Call GetCommand macro “CSwitchRange0”.  Use a two second delay to allow for hardware change.

Call GetCommand macro “HPMValue”.  Manipulate the response by stripping all characters up to the third appearance of a semi-colon.  The 761 return string echoes the command line with the measurement appended to the end.  See inset below.

Edit Commands, “Low P Sensor”:

Call GetCommand macro “CSwitchRange1”.  Use a two second delay to allow for hardware change.

Call GetCommand macro “HPMValue”.  Manipulate the response by stripping all characters up to the third appearance of a semi-colon.

Macros:  Create new GetCommand macros.

'**************************************************

'Appends a HEX NULL character ("0") to the end

'of the command string.

'The Additel requires a nonstandard terminating character instead

'of the traditional <CR>,<LF> options.

'This macro is called in lieu of the normal INITIALIZE command string.

'**************************************************

Function CSwitchRange0(Target, ChnlSt, SetMode, ParamID, cDevice)

CSwitchRange0 = "001:W:CSWITCHRANGE:0" & Chr(0)

End Function

'**************************************************

'Appends a HEX NULL character ("0") to the end

'of the command string.

'The Additel requires a nonstandard terminating character instead

'of the traditional <CR>,<LF> options.

'This macro is called in lieu of the normal INITIALIZE command string.

'**************************************************

Function CSwitchRange1(Target, ChnlSt, SetMode, ParamID, cDevice)

CSwitchRange1 = "001:W:CSWITCHRANGE:1" & Chr(0)

End Function

'**************************************************

'Appends a HEX NULL character ("0") to the end

'of the command string.

'The Additel requires a nonstandard terminating character instead

'of the traditional <CR>,<LF> options.

'This macro is called in lieu of the normal READ command string.

'**************************************************

Function HPMValue(Target, ChnlSt, SetMode, ParamID, cDevice)

HPMValue = "001:R:HPMVALUE" & Chr(0)

End Function

'**************************************************

'Appends a HEX NULL character ("0") to the end

'of the command string.

'The Additel requires a nonstandard terminating character instead

'of the traditional <CR>,<LF> options.

'This macro is called in lieu of the normal READ command string.

'**************************************************

Function LPMValue(Target, ChnlSt, SetMode, ParamID, cDevice)

LPMValue = "001:R:LPMVALUE" & Chr(0)

End Function